高爾夫球木桿之振動與碰撞分析

## Vibration and Impact Analysis of Golf Driver

研究生：吳國禎  Wu, Guo-Zhen

指導教授：王栢村  Wang, Bor-Tsuen

【摘要】

本論文旨在發展一套高爾夫球桿之球桿性能指標的評估模式與設計準則，並建立其標準分析作業流程。本論文以有限元素分析軟體導入電腦輔助工程分析(CAE)技術建立理論分析模式，分別進行其理論模態分析求得結構之自然頻率與及其模態振型，以及簡諧響應分析以求得頻率響應函數。另外也將實際高爾夫球桿結構進行完整之實驗模態分析(EMA)，求得實際結構之模態參數，以作為理論分析模型驗證與認證之參考據。完成模型驗證可確認分析模型之正確性，除探討高爾夫球桿結構振動特性之物理意義，可作為設計變更之參考，也將結合CAE與EMA技術導入虛擬測試之整合設計理念，如探討三種不同握把處邊界高爾夫球木桿模態特性之差異及建立以振動特性為基礎之高爾夫球桿之性能指標之評估模式，包括模態振型累加及ODS累加之打擊面型能指標與模態振型累加之握把處性能指標。並也建立應變量測分析的完整流程，利用套裝軟體ANSYS/LS-DYNA模擬高爾夫球以自由落體與球頭碰撞分析，並也實際對球頭進行應變量測。最後，利用上述所建立之理論動態碰撞分析，進行球頭砲擊測試，由理論分析進行應力預測，探討以理論預測球頭受砲擊後的應力分佈與球頭的變形狀況。本文建立之分析及實驗技術，以及虛擬測試之整合技術理念，將可拓展應用到其他產品之設計開發，可縮短研發時程與成本，提昇產品性能與品質。

關鍵字：有限元素分析、實驗模態分析、振動特性、模態參數、性能指標、模型驗證、虛擬測試

【Abstract】

This work presents the evaluation on performance index of the golf club wood driver and the standard operational procedure for design analysis. This work also introduces computer aided engineering (CAE) technique that utilizes the finite element commercial software to perform theoretical analysis. Both modal analysis and harmonic response analysis are carried out to obtain the structural modal parameters and frequency response functions, respectively. Besides, experimental modal analysis (EMA) is performed to determine the modal properties of the real structure. The experimentally extracted modal data are then used as the reference to proceed model verification and validation (V&V) process. Upon the completion of model V&V, the analytical model can be used to study dynamic characteristics of the golf club wood driver. The integrated design process via virtual testing is also presented to discuss the difference of modal characteristics of the golf club in different boundary conditions. Three types of performance indices of the club base on the vibration characteristics are defined and used to evaluate the club quality. The strain measurement technique is also built up to obtain the surface strain of the hit-surface of the golf club head due to the impact of the free-fall ball. Finally, the high speed ball impact on the golf club head can then be simulated to predict the stress distribution during impact. The proposed analytical and experimental methodology can be beneficial to the design and performance evaluation of golf club. The idea of virtual testing can be enhanced for practical design application.

Keywords：CAE, EMA, performance index, V&V, virtual testing